# Internet — Full Deep Explanation

## 🔹 What is the Internet?

The Internet is a massive global system of interconnected networks consisting of:

- Millions of routers

- Data centers

- Servers

- Fiber-optic cables

- Wireless links

- ISPs (Internet Service Providers)

It uses packet switching, where data is divided into small packets and routed independently.

## 🔹 Internet Hierarchy

### Tier-1 ISPs

- Global backbone providers

- Own international fiber routes

- Examples: AT&T, Level3, Tata Communications

### Tier-2 ISPs

- Regional providers

- Buy bandwidth from Tier-1

- Examples: National ISPs

### Tier-3 ISPs

- Local broadband ISPs

- They deliver internet to homes/offices

## 🔹 Key Components of the Internet

- DNS → Converts domain names to IP addresses

- BGP (Border Gateway Protocol) → Decides how packets travel globally

- CDN (Cloudflare, Akamai) → Deliver content fast globally

- IXP (Internet Exchange Point) → ISPs interconnect here

- Undersea Fiber Cables → Backbone of the Internet

# ARP Protocol — Deep Explanation

ARP (Address Resolution Protocol) maps an IP address → MAC address inside a LAN.

### Why ARP?

Because switches deliver frames using MAC, not IP.

### ARP Message Types

- ARP Request (Broadcast)
→ Who has 192.168.1.10?

- ARP Reply (Unicast)
→ I am 192.168.1.10, MAC = AA:BB:CC:DD:EE:FF

### ARP Cache Example

192.168.0.1   00:11:22:33:44:55   dynamic

192.168.0.10  FC:FB:FB:AA:BB:CC   static

### ARP Attacks

- ARP Spoofing / Poisoning

- Man-in-the-Middle

- Used in Kali Linux: Ettercap, Bettercap, arpspoof

# ICMP Protocol — Deep Explanation

ICMP = Internet Control Message Protocol
Used for error reporting and diagnostics.

### ICMP Works at:

- Network Layer (Layer 3)

### Common ICMP Types

| Type | Description |
| --- | --- |
| 0 | Echo Reply (Ping Reply) |
| 8 | Echo Request |
| 3 | Destination Unreachable |
| 11 | Time Exceeded |
| 5 | Redirect Message |

### ICMP Uses:

- Ping

- Traceroute

- Error handling

- Network troubleshooting

### Security Note:

Many firewalls block ICMP to prevent:

- Ping Flood

- Smurf Attack

# IGMP Protocol — Deep Explanation

IGMP (Internet Group Management Protocol) manages Multicast group memberships.

### Why Multicast?

Efficient delivery of:

- IPTV

- Video streaming

- Online seminars

- Live gaming sessions

### Multicast IP Range:

224.0.0.0 – 239.255.255.255

### IGMP Versions:

- IGMPv1 – Basic join

- IGMPv2 – Leave messages added

- IGMPv3 – Source-specific multicast (more secure)

# QUIC Protocol — Deep Explanation

QUIC = Quick UDP Internet Connections
Created by Google.

Used by HTTP/3.

### QUIC Features:

- Zero-RTT connection (ultra-fast)

- Built-in encryption (TLS 1.3)

- Works over UDP, but reliable

- No Head-of-Line Blocking

- Connection migration (WiFi → Mobile Data without disconnect)

### Who Uses QUIC?

- Google

- YouTube

- Facebook

- Cloudflare

- All HTTP/3 websites

# Ping Protocol — Deep Explanation

Ping is not a protocol itself.
It uses ICMP Echo Request/Reply messages.

### Ping Measures:

- Connection availability

- Round Trip Time (RTT)

- Packet loss

- Jitter

- DNS resolution

### Example:

64 bytes from 8.8.8.8: icmp_seq=1 ttl=118 time=32.1 ms

MAC Address — Deep Explanation

MAC = Media Access Control Address
A 48-bit, globally unique identifier for a network card.

### MAC Format:

AA:BB:CC:11:22:33

### Structure:

- First 24 bits: OUI (Manufacturer code)

- Last 24 bits: Device specific

### MAC Spoofing Example (Kali Linux)

ifconfig wlan0 down

macchanger -r wlan0

ifconfig wlan0 up

# Network Devices — Deep Technical Explanation

## Hub

- Layer 1 device

- Broadcasts all data

- No intelligence

- No MAC table

- Single collision domain

- Uses CSMA/CD

Hub = OLD + INSECURE technology.

## Switch

Layer 2 device (sometimes Layer 3).

### Switch Features:

- MAC Address Table

- STP (Spanning Tree Protocol)

- VLAN support

- Inter-VLAN routing (Layer 3 switch)

- QoS

- DHCP Snooping

- ARP Inspection

- Trunk links (802.1Q)

- Port Security

Switch = Smart + Secure + Fast.

## Router

Layer 3 device.

### Router Functions:

- IP Routing

- NAT/PAT

- DHCP Server

- Firewall

- VPN Tunneling

- Interconnecting networks

- Access Control Lists (ACL)

### Routing Protocols:

- OSPF, RIP, EIGRP → IGP

- BGP → Global Internet routing

## Bridge

- Older form of switch

- Layer 2

- Divides collision domains

- Slow, replaced by modern switches

## Repeater

- Layer 1

- Regenerates weak signals

- Used in fiber and WiFi boosters

## Gateway

Converts one protocol format → another protocol format.

Examples:

- VoIP Gateway → Converts SIP to PSTN

- Payment Gateway → Handles financial transactions

- IoT Gateway → Converts industrial protocols

A router can act as a gateway, but a gateway is more general.

## Access Point

Creates wireless networks (WiFi).

### Types:

- Standalone AP

- Controller-based AP

- Mesh AP

### AP Features:

- WPA2/WPA3 security

- WiFi 5/6/6E support

- Multi-SSID

- Channel selection

- Band steering

- Load balancing

## Modem

Modulator + Demodulator.

Converts ISP signal into digital data for routers.

Types:

- DSL Modem

- Cable Modem

- Fiber ONT (GPON/EPON)

# Synchronous & Asynchronous Data Transmission

## 🔹 Synchronous Transmission

Data is sent with a common clock signal.
Sender and receiver stay synchronized.

### Features:

- Continuous stream

- High speed

- Requires clock sync

- Used in: LAN, USB, DDR RAM, High-speed links

### Diagram:

Clock: | | | | | | | |

Data : 1 0 1 1 0 0 1

## 🔹 Asynchronous Transmission

Data is sent character-by-character without a clock signal.
Start/Stop bits indicate beginning and end.

### Features:

- Slower

- Simple & low cost

- No clock sync needed

- Used in: Keyboard, Mouse, Serial Port (RS-232)

### Diagram:

Start | Data Bits | Stop

0       10101      1

## 🔹 Difference Table

| Feature | Synchronous | Asynchronous |
| --- | --- | --- |
| Clock | Required | Not required |
| Speed | High | Low |
| Cost | Higher | Lower |
| Data Flow | Continuous | Byte-by-byte |
| Examples | DDR, USB, LAN | Keyboard, Serial port |

# Simplex, Half Duplex, Full Duplex

## 🔹 Simplex Mode

One-way communication.

Examples:

- TV broadcast

- Keyboard → PC (old systems)

Diagram:

A → B

## 🔹 Half Duplex Mode

Two-way communication, but one direction at a time.

Examples:

- Walkie-talkie

- Older wireless systems

Diagram:

A ↔ B  (one at a time)

## 🔹 Full Duplex Mode

Two-way communication simultaneously.

Examples:

- Telephone

- Modern Ethernet

- Switch communication

Diagram:

A ⇄ B  (simultaneous)

## 🔹 Difference Table

| Mode | Direction | Example |
| --- | --- | --- |
| Simplex | One-way | TV |
| Half Duplex | Both ways (not same time) | Walkie-talkie |
| Full Duplex | Both ways (same time) | Phone call |

# IP Addressing (IPv4 & IPv6)

# 🔹 IPv4 (32-bit)

- Length: 32 bits

- Format: Dotted decimal

- Example: 192.168.1.1

- Total: 4.3 billion addresses

### IPv4 Classes:

| Class | Range | Usage |
| --- | --- | --- |
| A | 1–126 | Large networks |
| B | 128–191 | Medium networks |
| C | 192–223 | Small networks |
| D | 224–239 | Multicast |
| E | 240–255 | Experimental |

## 🔹 IPv6 (128-bit)

- Length: 128 bits

- Format: Hexadecimal

- Example:
2001:0db8:85a3:0000:0000:8a2e:0370:7334

- Total: 340 undecillion addresses

### IPv6 Features:

- No NAT needed

- Auto-configuration (SLAAC)

- Built-in IPsec

- No broadcast (only multicast/anycast)

- Faster routing

# Subnetting — Full Explanation

Subnetting = dividing a large network into smaller sub-networks.

### Why Subnet?

- Better management

- Reduce broadcast traffic

- Improve security

- Efficient IP usage

### Example:

Given:
192.168.1.0/24 → 256 addresses

Divide into 2 subnets:

- 192.168.1.0/25

- 192.168.1.128/25

Each subnet:

- 128 IP addresses

- 126 usable

- 1 network + 1 broadcast

# Supernetting — Full Explanation

Supernetting = combining multiple small networks into one large block.
Also called Route Aggregation.

Used by ISPs for global routing.

### Example:

Combine:

- 192.168.0.0/24

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

→ Supernet = 192.168.0.0/22

# CIDR Notation — Full Explanation

CIDR = Classless Inter-Domain Routing

Format: IP/prefix

Examples:

- 10.0.0.0/8

- 192.168.1.0/24

- 172.16.0.0/12

Prefix length = network portion.

### Common CIDR Blocks:

| CIDR | Total IPs | Usable |
| --- | --- | --- |
| /8 | 16,777,216 | huge |
| /16 | 65,536 | large |
| /24 | 256 | small |
| /30 | 4 | point-to-point |
| /32 | 1 | host route |

# OSI Model — Full Explanation

OSI = Open Systems Interconnection
7 layers.

| Layer | Name | Function | Examples |
| --- | --- | --- | --- |
| 7 | Application | Services to user | HTTP, DNS |
| 6 | Presentation | Encryption, formatting | SSL/TLS |
| 5 | Session | Dialog control | Session IDs |
| 4 | Transport | End-to-end delivery | TCP, UDP |
| 3 | Network | Routing | IP, ICMP |
| 2 | Data Link | MAC, frames | Switch, ARP |
| 1 | Physical | Bits, signals | Cable, Hub |

# TCP/IP Model — Full Explanation

4 layers only.

| TCP/IP Layer | OSI Equivalent | Protocols |
| --- | --- | --- |
| Application | 5,6,7 | HTTP, FTP, DNS, SMTP |
| Transport | 4 | TCP, UDP |
| Internet | 3 | IP, ICMP, ARP |
| Network Access | 1,2 | Ethernet, WiFi |

# Well-Known Ports and Their Functions

These are standard ports assigned to specific network services.

## Most Important Ports (Table)

| Port | Protocol | Service | Function |
| --- | --- | --- | --- |
| 20/21 | TCP | FTP | File transfer |
| 22 | TCP | SSH | Secure remote login |
| 23 | TCP | Telnet | Unsecured remote login |
| 25 | TCP | SMTP | Sending emails |
| 53 | UDP/TCP | DNS | Domain → IP resolution |
| 67/68 | UDP | DHCP | Auto IP assignment |
| 69 | UDP | TFTP | Simple file transfer |
| 80 | TCP | HTTP | Web browsing |
| 110 | TCP | POP3 | Receive emails |
| 123 | UDP | NTP | Time synchronization |
| 143 | TCP | IMAP | Email retrieval |
| 161/162 | UDP | SNMP | Network monitoring |
| 389 | TCP/UDP | LDAP | Directory services |
| 443 | TCP | HTTPS | Secure web browsing |
| 445 | TCP | SMB | Windows file sharing |
| 514 | UDP | Syslog | Logging |
| 5060 | UDP/TCP | SIP | VoIP signaling |
| 8080 | TCP | HTTP Proxy | Web proxy |

# TCP & UDP Protocols (When, Where & How They Send Data)

## 🔹 TCP (Transmission Control Protocol)

### ✔ Where is TCP used?

Anywhere reliable, ordered, error-free communication is required:

- HTTPS (secure browsing)

- Email

- File Transfer (FTP)

- SSH remote login

- Database communication

### ✔ How TCP sends data?

- Connection-oriented

- Uses 3-way handshake

- Guarantees ordered data

- Retransmits lost packets

- Provides flow control & congestion control

TCP ensures:
➡ Data is delivered
➡ Data is in the correct order
➡ No data corruption

## 🔹 UDP (User Datagram Protocol)

### ✔ Where is UDP used?

Where speed is more important than reliability:

- Video streaming

- Online gaming

- VoIP calls

- DNS queries

- Live broadcasts

### ✔ How UDP sends data?

- Connectionless

- No handshake

- No delivery guarantee

- Fast and lightweight

UDP ensures:
➡ Very low latency
➡ Fast transmission
➡ Allows broadcast & multicast

## 🔹 Difference Table

| Feature | TCP | UDP |
| --- | --- | --- |
| Type | Connection-oriented | Connectionless |
| Speed | Slow | Fast |
| Reliability | High | Low |
| Ordering | Guaranteed | Not guaranteed |
| Use cases | Web, email | Gaming, live streaming |

# TCP 3-Way Handshake (Connection Establishment)

Used to establish a TCP connection.

Client ---- SYN ---> Server

Client <--- SYN/ACK --- Server

Client ---- ACK ---> Server

### Step-by-step:

- SYN → Client: “I want to connect.”

- SYN/ACK → Server: “I accept.”

- ACK → Client: “Okay, connection ready.”

Connection established successfully.

# TCP 4-Way Handshake (Connection Termination)

Used when closing a TCP connection.

Client ---- FIN ---> Server

Client <--- ACK ---- Server

Client <--- FIN ---- Server

Client ---- ACK ---> Server

### Step-by-step:

- Client sends FIN

- Server replies ACK

- Server sends its own FIN

- Client replies ACK

Both sides close gracefully.

# TCP Flags (SYN, ACK, FIN, RST, PSH, URG)

Flags identify the purpose of a TCP packet.

| Flag | Full Form | Meaning |
| --- | --- | --- |
| SYN | Synchronize | Start connection |
| ACK | Acknowledgment | Confirm packet received |
| FIN | Finish | Close connection |
| RST | Reset | Force close connection |
| PSH | Push | Send data immediately (skip buffer) |
| URG | Urgent | High-priority data |

### Usage Example:

- SYN = Start handshake

- SYN/ACK = Accept handshake

- FIN = Initiate close

- RST = Emergency termination

# HTTP Methods (GET, POST, PUT, PATCH, TRACE, HEAD, OPTIONS)

HTTP methods are commands used by a client to interact with a web server.

## HTTP Methods Table

| Method | Purpose | Description |
| --- | --- | --- |
| GET | Retrieve data | Data visible in URL, safe for read-only |
| POST | Submit data | Used for login, signup, form submission |
| PUT | Replace resource | Full update of a record |
| PATCH | Partial update | Modify only specific fields |
| DELETE | Remove resource | Delete an item |
| HEAD | Get headers | Same as GET but without body |
| OPTIONS | Check supported methods | Used for CORS, debugging |
| TRACE | Loopback test | Shows request path (usually disabled) |

### 🔹 Short Explanation

### GET

- Used to request data

- Visible in URL

- Not used for sensitive info

### POST

- Sends data in body

- Secure for login/password

- Used in most forms

### PUT

- Replaces entire record

### PATCH

- Updates only part of the resource

### HEAD

- Checks if resource exists

### OPTIONS

- Shows allowed HTTP methods (important for CORS)

### TRACE

- Returns the request path (security risk, often turned off)

# DNS (Domain Name System)

DNS works as the phonebook of the internet.
It converts domain names → IP addresses.

### Example:

google.com → 142.250.190.78

facebook.com → 157.240.200.35

### 🔹 How DNS Works (Steps)

- User types domain

- Browser sends DNS query

- Resolver checks cache

- Goes to Root Server

- Root → TLD Server (.com, .net, etc.)

- TLD → Authoritative DNS

- Authoritative DNS returns IP

- Browser connects to the server

### 🔹 DNS Record Types

| Record | Purpose |
| --- | --- |
| A | IPv4 Address |
| AAAA | IPv6 Address |
| CNAME | Alias name |
| MX | Mail server |
| TXT | Verification, SPF/DKIM |
| NS | Name server |
| PTR | Reverse DNS |

# DHCP (Dynamic Host Configuration Protocol)

DHCP automatically assigns:

- IP Address

- Subnet Mask

- Gateway

- DNS Server

- Lease Time

### 🔹 DHCP Process (DORA)

- Discover

- Offer

- Request

- Acknowledge

# FTP, TFTP, SFTP, SCP

## 🔹 FTP (File Transfer Protocol)

- Port: 20/21

- Not encrypted (unsafe)

- Upload/download files

## 🔹 TFTP (Trivial FTP)

- Port: 69

- No authentication

- Very fast

- Used for router booting, PXE boot

## 🔹 SFTP (SSH File Transfer Protocol)

- Port: 22

- Secure (SSH-based)

- Secure file uploads/downloads

## 🔹 SCP (Secure Copy Protocol)

- Port: 22

- Very fast secure copying over SSH

# SSH & Telnet

## 🔹 Telnet

- Port: 23

- No encryption

- Sends data as plain text

- Not recommended

## 🔹 SSH (Secure Shell)

- Port: 22

- Fully encrypted

- Used for safe remote login

- Used by admins, servers, hackers (pentesting)

# SMTP, POP3, IMAP

## 🔹 SMTP (Simple Mail Transfer Protocol)

- Port: 25

- Used to send emails

## 🔹 POP3 (Post Office Protocol v3)

- Port: 110

- Downloads emails to device

- Often deletes from server

## 🔹 IMAP (Internet Message Access Protocol)

- Port: 143

- Sync emails across devices

- Keeps mail on server

# SNMP (Simple Network Management Protocol)

Used for network monitoring and management.

### SNMP Monitors:

- Router/Switch health

- Traffic usage

- Errors

- CPU load

- Interface up/down

### Ports:

- 161 – Queries

- 162 – Alerts (Traps)

# NTP (Network Time Protocol)

- Port: 123 (UDP)

- Synchronizes time across devices

- Used in servers, routers, banks, security systems

Example:
Google NTP → time.google.com

# SMB, LDAP, RDP

## 🔹 SMB (Server Message Block)

- Port: 445

- Windows File Sharing

- Printer sharing

- AD communication

- Vulnerable to: EternalBlue, WannaCry

## 🔹 LDAP (Lightweight Directory Access Protocol)

- Port: 389

- Used for authentication & directory lookups

- Used in Active Directory (AD)

## 🔹 RDP (Remote Desktop Protocol)

- Port: 3389

- Used for Windows remote access

- Full graphical remote control

# NAT & PAT (Network and Port Address Translation)

## 🔹 NAT

Translates private IP → public IP.

Example:

192.168.1.5 → 203.0.113.10

## 🔹 PAT (NAT Overload)

Multiple private devices share ONE public IP using different port numbers.

Example:

192.168.1.10:5050 → 203.0.113.10:15501

192.168.1.11:5050 → 203.0.113.10:15502

Used in home routers.

# Firewalls & CDN

## 🔹 Firewall

Controls and filters traffic based on:

- IP

- Port

- Protocol

- Application

- URL

- Content

- Behavior

### Types:

- Packet-filtering firewall

- Stateful firewall

- Proxy firewall

- NGFW (Next-Gen Firewall)

## 🔹 CDN (Content Delivery Network)

CDN delivers content fast across the world using:

- Global caching

- Load balancing

- DDoS protection

- Smart routing

Examples:
Cloudflare, Akamai, Fastly

# VPN Types (SSL, IPsec, PPTP, L2TP)

## 🔹 SSL VPN

- Runs over HTTPS (443)

- Easy to use

- Perfect for remote workers

## 🔹 IPsec VPN

- Most secure

- Works at Network Layer

- Used for site-to-site VPN

- Strong encryption

## 🔹 PPTP

- Very old

- Weak encryption

- Not recommended

## 🔹 L2TP/IPsec

- Combines L2TP + IPsec

- More secure + stable

- Widely used in enterprise environments

# TLS (Transport Layer Security)

TLS = Security layer for HTTPS.

### TLS Provides:

- Encryption (data secure)

- Authentication (server verified)

- Integrity (data not changed)

- Protection from MITM attacks

### Versions:

- TLS 1.0 → Deprecated

- TLS 1.2 → Standard

- TLS 1.3 → Fastest, Most Secure

# Netstat & Traceroute

## 🔹 Netstat

Netstat = Network Statistics
Used to view all network connections in a system.

### Netstat Shows:

- Active TCP/UDP connections

- Listening ports

- Established connections

- Routing table

- Interfaces

### Common Command:

netstat -tulnp

| Flag | Meaning |
| --- | --- |
| -t | TCP |
| -u | UDP |
| -l | Listening |
| -n | Numeric IP/ports |
| -p | Process ID |

## 🔹 Traceroute

Traceroute shows the path (hops) a packet takes to reach its destination.

### How it works?

Traceroute increases TTL step-by-step:

- TTL=1 → 1st router responds

- TTL=2 → 2nd router responds

Command:

traceroute google.com

# Encryption Protocols (WEP, WPA, WPA2, WPA3)

## 🔹 WEP

- Very weak

- Uses RC4

- Easily cracked

- Deprecated

## 🔹 WPA

- Uses TKIP

- More secure than WEP

- Still weak (WPS attack possible)

## 🔹 WPA2

- Most common

- Uses AES-CCMP

- Strong security

## 🔹 WPA3

- Uses SAE (Dragonfly handshake)

- Resists KRACK

- Strongest WiFi security

# Wireless Authentication (PSK, Enterprise)

## 🔹 PSK (Pre-Shared Key)

- Shared WiFi password

- Used in home networks

- Everyone uses same key

## 🔹 Enterprise (802.1X + RADIUS)

- Username & password login

- Uses RADIUS server

- Used in organizations

### Comparison:

| Feature | PSK | Enterprise |
| --- | --- | --- |
| Key | Shared | Per user |
| Security | Medium | High |
| Use | Home | Corporate |

# Wireless Attacks (Deauth, Evil Twin, KRACK)

## 🔹 1) Deauthentication Attack

- Sends fake deauth frames

- Disconnects users from WiFi

- Used to capture WPA handshake

- Tool: aireplay-ng

## 🔹 2) Evil Twin

- Creates a fake Access Point

- Victim connects to fake AP

- Attacker performs MITM

- Used to steal credentials

## 🔹 3) KRACK Attack

- Weakness in WPA2 handshake

- Attacker decrypts traffic

- Works using key reinstallation attacks

# ARP Spoofing / ARP Poisoning

Attacker sends fake ARP replies to poison the ARP table.

### Goal:

Redirect victim’s traffic through attacker.

### Results:

- MITM

- Sniffing passwords

- DNS spoofing

- Session hijacking

### Tools:

- Ettercap

- Bettercap

- arpspoof

# DNS Spoofing

Fake DNS responses to redirect a victim to a malicious IP.

### Example:

facebook.com → 192.168.1.50 (attacker)

Used for:

- Phishing

- Credential theft

- MITM

# IP Spoofing

Attacker sends packets with a fake source IP address.

### Used for:

- Hiding identity

- SYN flood attacks

- Reflection/Amplification attacks

- Bypassing IP-based filters

# MAC Flooding

Attacker floods the switch with random MAC addresses.

### Effect:

- Switch MAC table becomes full

- Switch enters hub mode

- Attacker can sniff all traffic

### Tool:

macof (Dsniff package)

# DHCP Starvation

Attacker sends many fake DHCP discover requests.

### Result:

- DHCP server runs out of IP addresses

- Legit users cannot get IP

- Attacker sets up rogue DHCP server

### Tool:

Yersinia

# SYN Flood / DoS / DDoS

## 🔹 SYN Flood

Attacker sends huge SYN packets → never completes handshake.

### Result:

- Server resources exhausted

- Half-open table full

- Server crashes or slows down

## 🔹 DoS

One attacker → floods a target.

## 🔹 DDoS

Many distributed attackers (botnets) flood a target.

Botnets examples:

- Mirai

- Zeus

# MITM (Man-in-the-Middle)

Attacker sits between two communicating parties.

### Techniques:

- ARP Spoofing

- DNS Spoofing

- Evil Twin

- SSL Stripping

- HTTPS Downgrade

### Attacker can:

- Read data

- Modify data

- Steal credentials

- Inject malicious scripts

# Sniffing & Eavesdropping

## 🔹 Sniffing

Sniffing means capturing network traffic to read data packets.

### Types:

### ✔ Passive Sniffing

- Attacker only listens

- Does not modify traffic

- Works in hub/broadcast networks

### ✔ Active Sniffing

- Attacker manipulates traffic

- Works in switched networks

- Techniques:

  - ARP Poisoning

  - MAC Flooding

  - DHCP Spoofing

### Tools:

- Wireshark

- tcpdump

- Bettercap

- Ettercap

## 🔹 Eavesdropping

Secretly listening to communication.

In network security:

- Happens when traffic is unencrypted

- Attacker steals usernames, passwords, cookies

Eavesdropping = covert sniffing.

# Replay Attacks

Attacker captures legitimate packets and re-sends them later.

### Examples:

- Replaying login requests

- Repeating financial transaction packets

- Reusing authentication tokens

### Prevention:

- Nonce

- Time-stamps

- Session tokens

- TLS/HTTPS

# Session Hijacking

Attacker takes over an active user session.

### Methods:

- Cookie hijacking

- Session ID prediction

- MITM-based session takeover

### Steps:

- Capture session cookie

- Use cookie to log in as victim

### Tools:

- Burp Suite

- Bettercap

- Wireshark

# ICMP Flood, Ping of Death, Smurf Attack

## 🔹 ICMP Flood

Attacker sends massive ICMP Echo (ping) requests.

Result:

- Target CPU overload

- Network congestion

- DoS condition

## 🔹 Ping of Death

- Sends oversized ICMP packets

- Causes crash or overflow

- Old systems vulnerable

## 🔹 Smurf Attack

- Attacker spoofs victim’s IP

- Sends ICMP to broadcast address

- Whole network replies to victim

- Victim gets overwhelmed

# VLAN Hopping

Attacker jumps from one VLAN to another VLAN.

### Techniques:

- Switch Spoofing

  - Attacker pretends to be a trunk port

- Double Tagging

  - Uses 2 VLAN tags to bypass VLAN boundaries

### Prevention:

- Disable DTP

- Set all ports to access mode

- Use VLAN pruning

# Rogue DHCP & Evil Twin

## 🔹 Rogue DHCP

Attacker creates a fake DHCP server.

### Victim receives:

- Wrong gateway

- Wrong DNS

- Wrong IP

Attacker can then:

- Redirect traffic

- Perform MITM

- Conduct DNS spoofing

## 🔹 Evil Twin

A fake WiFi Access Point identical to the legitimate one.

### Uses:

- MITM

- Credential theft

- Packet sniffing

Tools:

- Airbase-ng

- hostapd-wpe

# Routing Attacks

Routing attacks target routing tables or routing protocols.

### Types:

### ✔ Route Injection

Attacker injects fake routes.

### ✔ Blackhole Attack

Attacker absorbs/drops all traffic.

### ✔ Routing Table Poisoning

Modifies routing entries with malicious information.

### Targeted protocols:

- BGP

- OSPF

- RIP

# SSL Stripping

Attacker downgrades HTTPS → HTTP to steal data.

### How it works:

- Victim sends HTTPS request

- Attacker intercepts

- Attacker connects HTTPS to server

- Victim receives HTTP

- Attacker reads and modifies everything

Tools:

- sslstrip

- Bettercap

- Burp Suite

# Port Scanning Detection & Prevention

## Detection:

- IDS/IPS (Snort, Suricata)

- Firewall logs

- Abnormal SYN patterns

- Nmap activity detection

## Prevention:

- SYN cookies

- Port knocking

- Disable unnecessary services

- Rate limiting

- Strong firewall rules

# Pivoting Through Networks

After compromising one machine, attacker uses it to access internal networks.

### Techniques:

- SSH pivoting

- SOCKS proxy

- Proxychains

- Metasploit route add

- VPN tunneling

Example:

Attacker → Compromised PC → Internal servers

# Traffic Redirection & Tunneling

## 🔹 Traffic Redirection

Attacker redirects victim traffic:

- DNS spoofing

- ARP poisoning

- Route manipulation

## 🔹 Tunneling

Hiding one protocol inside another to bypass firewalls.

Examples:

- SSH tunnel

- HTTP tunnel

- DNS tunnel

- ICMP tunnel

Used for bypassing restrictions.

# Reverse Shells & Bind Shells

## 🔹 Reverse Shell

Victim connects back to attacker.

Victim → Attacker listener

Used because:

- Firewalls often block inbound traffic

- But allow outbound traffic

Tools:

- Netcat

- Metasploit

- Python payloads

## 🔹 Bind Shell

Victim opens a port and attacker connects.

Attacker → Victim open port

Harder to bypass firewalls.