# Internet (ইন্টারনেট)

## 🔹 ইন্টারনেট আসলে কী?

ইন্টারনেট হলো:

- বিশ্বজুড়ে অসংখ্য ডিভাইসের

- কোটি কোটি রাউটার, সার্ভার, অপটিক ফাইবার লিঙ্কের

- Interconnected network

এই নেটওয়ার্কগুলো BGP (Border Gateway Protocol) নামের routing protocol ব্যবহার করে একে অপরের মধ্যে পথ ঠিক করে।

## 🔹 ইন্টারনেট কিভাবে চলে?

### Internet Structure:

#### 1) Tier-1 ISP

- Global-level providers

- Example: Level 3, Tata, AT&T

- এরা world-wide backbone নেটওয়ার্ক পরিচালনা করে

#### 2) Tier-2 ISP

- Large regional ISP

- Example: Banglalink, GP, Airtel

- এরা Tier-1 থেকে bandwidth কিনে

#### 3) Tier-3 ISP

- Local broadband provider

- তোমার বাসায় যেই লাইন আসে

## 🔹 Internet Data Flow

- তুমি ব্রাউজারে google.com লিখলে

- DNS query যায় DNS সার্ভারে

- DNS → Google-এর public IP দেয়

- তোমার রাউটার থেকে ISP router

- ISP থেকে multiple backbone → Google data center

- সার্ভার উত্তর পাঠায়

- প্যাকেট ফিরে আসে একই বা ভিন্ন রুটে

### Important Components:

- DNS → Domain → IP রূপান্তর

- BGP → Internet routing control করে

- CDN (Cloudflare, Akamai) → Content world-wide দ্রুত পৌঁছায়

- Optical Fiber Backbone → Undersea cables

- IXP (Internet Exchange Point) → ISPs interconnect হয়

# ARP Protocol — Deep Explanation

### ARP কী করে?

LAN-এর মধ্যে IP → MAC পাওয়ার একমাত্র উপায়।

### ARP Message Types:

- ARP Request (Broadcast)
→ Everyone listen! Who has 192.168.1.10?

- ARP Reply (Unicast)
→ I am 192.168.1.10, my MAC is AA:BB:CC:DD:EE:FF

### ARP Cache/Table:

OS এর RAM-এ নিচের মত স্টোর হয়:

192.168.1.1    -> 00:11:22:33:44:55  (valid 60 sec)

### ARP Spoofing (Attack)

Attackers fake ARP Reply পাঠায়:

- Router’s IP → Attacker’s MAC

- Result = MITM attack

- Wireshark-এ দেখা যায়

# ICMP Protocol — Deep Explanation

### ICMP কোন Layer?

- Layer 3 (Network Layer)

- IP-এর ভিতরে encapsulated হয়

### ICMP কিসের জন্য?

- Error reporting

- Network testing

- Diagnostic

### Common ICMP Messages:

| ICMP Type | Meaning |
| --- | --- |
| 0 | Echo Reply (ping reply) |
| 8 | Echo Request |
| 3 | Destination Unreachable |
| 11 | Time Exceeded |
| 5 | Redirect |

### ICMP Security Note:

অনেক firewall ICMP block করে:

- DoS attack হতে পারে

- Ping flood

- Smurf attack

# IGMP Protocol — Deep Explanation

### IGMP = Multicast Membership Management

Multicast IP range:
224.0.0.0 – 239.255.255.255

### কেন multicast দরকার?

Unicast → 1:1
Broadcast → সবাই
Multicast → গ্রুপ-ভিত্তিক (efficient)

### IGMP versions:

- IGMPv1 – Basic join

- IGMPv2 – Leave message

- IGMPv3 – Source-specific multicast (SSM)

### উদাহরণ:

IPTV → Channel join/leave
Video conference → group video

# QUIC Protocol — Deep Explanation

### QUIC = Fast Reliable connection over UDP

UDP ব্যবহার করে কিন্তু TCP-এর ফিচারগুলা রাখা হয়েছে।

### QUIC-এর Features:

- Zero-RTT connection

- Built-in TLS 1.3

- No Head-of-Line Blocking

- Connection migration
(WiFi → Mobile data switch হলেও connection break হয় না)

### QUIC ব্যবহারকারীরা:

- HTTP/3 এ QUIC ব্যবহার বাধ্যতামূলক

- Google, Facebook, Cloudflare, YouTube

# Ping Protocol — Deep Explanation

Ping = ICMP Echo Request/Reply.

### Ping কি মাপতে পারে?

- RTT (Round Trip Time)

- Jitter

- Packet loss

- DNS resolution

- Network reachability

### Example Output:

64 bytes from 8.8.8.8: icmp_seq=1 ttl=118 time=32.1 ms

# MAC Address — Deep Explanation

### MAC structure (48-bit)

- 1st 24-bit → Manufacturer (OUI)

- Last 24-bit → Device serial

### Example:

FC:FB:FB:01:02:03

- FC:FB:FB → Apple

- Rest → Device specific

### MAC Filtering:

WiFi security → Allow list / Block list

### MAC Spoofing:

Kali Linux:

ifconfig wlan0 down

macchanger -r wlan0

ifconfig wlan0 up

# Network Devices — Full Deep Explanation

## Hub

- Pure physical layer device (Layer 1)

- Broadcasts all data

- No MAC table

- Collision domain = 1

- Very insecure

- Uses CSMA/CD

## Switch

Layer 2 device (sometimes Layer 3)

### Switch Features:

- MAC table

- VLAN

- STP (Spanning Tree Protocol)

- Trunking (802.1Q)

- EtherChannel (Port aggregation)

- QoS

- ARP Inspection

- DHCP Snooping

Switch frames forwarding:

- Reads MAC

- Checks table

- Forward/Filter/Flood

## Router

Layer 3 device।

### Router Features:

- Routing (static/dynamic)

- NAT/PAT

- DHCP server

- Firewall

- VPN termination

- Inter-VLAN routing

### Routing Protocols:

- IGP: OSPF, EIGRP, RIP

- EGP: BGP

## Bridge

Switch-এর পুরনো ভার্সন।

কাজ:

- Collision domain ভাগ করে

- MAC filtering

- Slow, replacing by switches

## Repeater

Layer 1 device
Signal regenerate করে
Range বাড়ায়
Used in fiber, WiFi boosters

## Gateway

Protocol converter:

উদাহরণ:

- VoIP gateway (SIP → PSTN)

- Payment gateway

- IoT gateway

## Access Point

WiFi network তৈরি করে।

### Types:

- Standalone AP

- Controller-based AP

- Mesh AP

### Features:

- WPA2/WPA3 encryption

- WiFi 6/6E

- SSID broadcasting

- Channel selection

- Band steering

- Load balancing

## Modem

Modulator + Demodulator
ISP line → Router

Types:

- DSL modem

- Cable modem

- Fiber ONT (GPON, EPON)

# Synchronous & Asynchronous Data Transmission

## 🔹 Synchronous Transmission

ডাটা পাঠানো হয় clock signal অনুযায়ী, অর্থাৎ sender–receiver-এর মধ্যে timing match থাকে।

### বৈশিষ্ট্য:

- Continuous data stream

- Fast transmission

- Clock sync প্রয়োজন

- Used in: LAN, High-speed networks, DDR RAM, USB

### Diagram (Conceptual):

Clock:  | | | | | | | |

Data :  1 0 1 1 0 0 1

## 🔹 Asynchronous Transmission

ডাটা পাঠানো হয় bit-by-bit ভিত্তিতে।
Start bit + Stop bit দিয়ে ডাটা বোঝায়।

### বৈশিষ্ট্য:

- No clock sync needed

- Slower

- Low-cost

- Used in: Keyboard, Mouse, Serial comm (RS-232)

### Diagram:

Start | Data Bits | Stop

0       10101      1

## 🔹 Difference Table

| Feature | Synchronous | Asynchronous |
| --- | --- | --- |
| Clock | Required | Not required |
| Speed | Fast | Slow |
| Cost | High | Low |
| Data flow | Stream | Byte-by-byte |
| Examples | USB, DDR, LAN | Keyboard, Serial port |

# Simplex, Half Duplex, Full Duplex

## 🔹 Simplex

ডাটা শুধু এক দিক দিয়ে যায়।

উদাহরণ:

- TV broadcasting

- Fire alarm system

Diagram:

A → B

## 🔹 Half Duplex

ডাটা দুই দিকেই যায়, কিন্তু একসময় এক দিক।

উদাহরণ:

- Walkie-Talkie

- Old wireless systems

Diagram:

A ↔ B  (not simultaneously)

## 🔹 Full Duplex

ডাটা দুই দিকেই যায় একসাথে।

উদাহরণ:

- Phone call

- Modern Computers

- Ethernet switches

Diagram:

A ⇄ B (simultaneously)

## 🔹 Difference Table

| Mode | Direction | Example |
| --- | --- | --- |
| Simplex | One-way | TV |
| Half Duplex | Both-way (not same time) | Walkie-talkie |
| Full Duplex | Both-way (same time) | Phone call |

# IP Addressing (IPv4 & IPv6)

# 🔹 IPv4 (32-bit)

- Length: 32-bit

- Format: Decimal

- Example: 192.168.1.10

- Total addresses: 4.3 billion

### IPv4 Classes:

| Class | Range | Use |
| --- | --- | --- |
| A | 1–126 | Large networks |
| B | 128–191 | Medium networks |
| C | 192–223 | Small networks |
| D | 224–239 | Multicast |
| E | 240–255 | Experimental |

# 🔹 IPv6 (128-bit)

- Length: 128-bit

- Format: Hexadecimal

- Example:
2001:0db8:85a3:0000:0000:8a2e:0370:7334

- Total addresses: 340 undecillion

- Auto-configuration supported (SLAAC)

### Features of IPv6:

- No NAT required

- Built-in IPsec

- Hierarchical addressing

- Faster routing

- No broadcast (only multicast & anycast)

# Subnetting — Full Explanation

Subnetting = বড় নেটওয়ার্ককে ছোট ছোট নেটওয়ার্কে ভাগ করা।

### কেন Subnetting করা হয়?

- IP save করা

- Network management সহজ

- Security বাড়ে

- Network performance বাড়ে

## Subnetting Example:

Given:
192.168.1.0/24 → 256 total IPs

If you divide into 2 subnets:

- 192.168.1.0/25

- 192.168.1.128/25

Each subnet contains:

- 128 IP

- Usable = 126

- Broadcast = last IP

# Supernetting — Full Explanation

Supernetting = ছোট network → বড় network combine করা।
Also called Route Aggregation.

Used by ISPs, BGP routing.

### Example:

Combine:

- 192.168.0.0/24

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

→ Supernet becomes:
192.168.0.0/22

# CIDR Notation — Full Explanation

CIDR = Classless Inter-Domain Routing
Format → IP/prefix

Example:

- 192.168.1.0/24

- 10.0.0.0/8

Prefix length says how many bits = network portion.

### Some Important CIDR blocks:

| CIDR | Total IP | Usable |
| --- | --- | --- |
| /8 | 16,777,216 | huge |
| /16 | 65,536 | medium |
| /24 | 256 | small |
| /30 | 4 | point-to-point |
| /32 | 1 | host route |

# OSI Model — Full Explanation

### OSI = Open Systems Interconnection

Total: 7 Layers

| Layer | Name | Function | Example |
| --- | --- | --- | --- |
| 7 | Application | User interface | HTTP, DNS |
| 6 | Presentation | Data format, encryption | SSL/TLS |
| 5 | Session | Connection control | Session keys |
| 4 | Transport | Reliable delivery | TCP/UDP |
| 3 | Network | Routing | IP |
| 2 | Data Link | MAC addressing | Switch |
| 1 | Physical | Bits, signals | Cable, Hub |

# TCP/IP Model — Full Explanation

4-layer model:

| TCP/IP Layer | OSI Equivalent | Examples |
| --- | --- | --- |
| Application | 5,6,7 | HTTP, FTP, DNS |
| Transport | 4 | TCP, UDP |
| Internet | 3 | IP, ICMP |
| Network Access | 1,2 | Ethernet, WiFi |

# Well-Known Ports and Their Works

এগুলো হলো Standard Ports যেগুলো নির্দিষ্ট সার্ভিসের জন্য ব্যবহৃত হয়।

## Most Important Ports (Table)

| Port | Protocol | Service | কাজ |
| --- | --- | --- | --- |
| 20/21 | TCP | FTP | File transfer |
| 22 | TCP | SSH | Secure remote login |
| 23 | TCP | Telnet | Unsecure remote login |
| 25 | TCP | SMTP | Email sending |
| 53 | TCP/UDP | DNS | Name resolution |
| 67/68 | UDP | DHCP | IP assign করা |
| 69 | UDP | TFTP | Simple file transfer |
| 80 | TCP | HTTP | Web browsing |
| 110 | TCP | POP3 | Receive email |
| 123 | UDP | NTP | Time sync |
| 143 | TCP | IMAP | Email retrieve |
| 161/162 | UDP | SNMP | Network monitoring |
| 389 | TCP/UDP | LDAP | Directory services |
| 443 | TCP | HTTPS | Secure browsing |
| 445 | TCP | SMB | File sharing (Windows) |
| 514 | UDP | Syslog | Logging |
| 5060 | UDP/TCP | SIP | VoIP |
| 8080 | TCP | HTTP Proxy | Web proxy |

# TCP & UDP Protocol (When, Where, How They Send Data)

## 🔹 TCP (Transmission Control Protocol)

### ✔ TCP কোথায় ব্যবহার হয়?

যেখানে reliable, error-free, ordered data দরকার:

- Web browsing (HTTPS)

- Email

- File transfer (FTP)

- Remote login (SSH)

- Database communication

### ✔ TCP কিভাবে ডাটা পাঠায়?

- Connection-oriented

- 3-way handshake দিয়ে শুরু

- Ordered delivery

- Lost packet retransmit করে

- Flow control (Windowing)

- Congestion control

TCP ensures:
➡ Data MUST reach
➡ Data MUST be in order
➡ No corruption allowed

## 🔹 UDP (User Datagram Protocol)

### ✔ UDP কোথায় ব্যবহার হয়?

যেখানে speed গুরুত্বপূর্ণ, reliability না:

- Video streaming

- Online gaming

- VoIP call

- DNS

- Live broadcasting

### ✔ UDP কিভাবে ডাটা পাঠায়?

- Connectionless

- No handshake

- No guarantee

- Very fast

- Lightweight packets

UDP ensures:
➡ Speed
➡ Low latency
➡ Broadcasting possible

## 🔹 Difference Table

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Yes | No |
| Speed | Slow | Fast |
| Reliability | High | Low |
| Ordering | Guaranteed | Not guaranteed |
| Use cases | Web, Email | Gaming, Live stream |

# TCP 3-Way Handshake (Connection Establishment)

TCP connection establish করার জন্য তিনটি ধাপ থাকে:

Client ---- SYN ---> Server

Client <--- SYN/ACK --- Server

Client ---- ACK ---> Server

## 🔹 Step-by-step:

- SYN
Client বলে: “I want to connect.”

- SYN + ACK
Server বলে: “I’m ready. You can connect.”

- ACK
Client বলে: “Okay, connection established.”

এটাই secure, reliable TCP connection শুরু হওয়ার পদ্ধতি।

# TCP 4-Way Handshake (Connection Termination)

TCP connection বন্ধ করার ৪ ধাপ থাকে:

Client ---- FIN ---> Server

Client <--- ACK ---- Server

Client <--- FIN ---- Server

Client ---- ACK ---> Server

## 🔹 Step-by-step:

- Client → FIN

- Server → ACK

- Server → FIN

- Client → ACK

Connection closes from both sides.

# TCP Flags (SYN, ACK, FIN, RST, PSH, URG)

TCP packet header-এ 6টি main flags থাকে।

| Flag | Meaning | কাজ |
| --- | --- | --- |
| SYN | Synchronize | Connection start |
| ACK | Acknowledgement | Packet received confirm |
| FIN | Finish | Connection close |
| RST | Reset | Forcefully close |
| PSH | Push | Urgent send (buffer bypass) |
| URG | Urgent | High priority packets |

### Example:

- SYN = handshake start

- SYN + ACK = handshake accept

- FIN = close request

- RST = sudden termination

# HTTP Methods (GET, POST, PUT, PATCH, TRACE, HEAD, OPTIONS)

এগুলো ওয়েব সার্ভারে বিভিন্ন উদ্দেশ্যে request পাঠায়।

## Table with Full Explanation

| Method | Use | Works |
| --- | --- | --- |
| GET | Data read | URL-এ ডাটা দেখা যায় |
| POST | Data submit | Form submission, login |
| PUT | Replace data | Full update |
| PATCH | Partial update | ছোট অংশ modify |
| DELETE | Remove data | Record delete |
| HEAD | Headers only | Resource exists কিনা দেখা |
| OPTIONS | Allowed methods | Server কোন HTTP methods সাপোর্ট করে |
| TRACE | Loopback test | Debugging request path |

### 🔹 Short Explanation:

### GET

- Only retrieve data

- Visible in URL

- Not secure for passwords

### POST

- Secure

- Body-তে data যায়

- Login, signup, upload

### PUT

- Entire record replace করে

### PATCH

- Only selected fields modify

### HEAD

- GET এর মতো কিন্তু body ছাড়া

### OPTIONS

- Check allowed methods (CORS)

### TRACE

- Security risk, often disabled

# DNS (Domain Name System)

DNS হলো ইন্টারনেটের ফোনবুক।
এটি ডোমেইন নাম → IP address-এ রূপান্তর করে।

### উদাহরণ:

google.com → 142.250.190.78

facebook.com → 157.240.200.35

### DNS কাজের ধাপ

- User টাইপ করে: google.com

- Browser sends DNS query

- Resolver → Root server

- Root → TLD server (.com)

- TLD → Authoritative DNS

- Authoritative DNS IP পাঠায়

- Browser সার্ভারে connect করে

### DNS Record Types:

| Type | কাজ |
| --- | --- |
| A | IPv4 address |
| AAAA | IPv6 address |
| CNAME | Alias name |
| MX | Mail server |
| TXT | Verification |
| NS | Nameserver |
| PTR | Reverse DNS |

# DHCP (Dynamic Host Configuration Protocol)

DHCP স্বয়ংক্রিয়ভাবে ডিভাইসকে নেটওয়ার্ক সেটিং দেয়।

### DHCP দেয়:

- IP Address

- Subnet Mask

- Gateway

- DNS Server

- Lease Time

### DHCP কাজের চার ধাপ (DORA):

- Discover

- Offer

- Request

- Acknowledge

# FTP, TFTP, SFTP, SCP

## 🔹 FTP (File Transfer Protocol)

- Port: 20/21

- Unsecure

- File upload/download

- Used in legacy systems

## 🔹 TFTP (Trivial FTP)

- Port: 69

- No authentication

- Fast & lightweight

- Used in: Booting routers, PXE boot

## 🔹 SFTP (SSH File Transfer Protocol)

- Port: 22

- Secure (SSH-based)

- Used in: Secure file transfers

## 🔹 SCP (Secure Copy Protocol)

- Port: 22

- Works over SSH

- Very fast secure copy

# SSH & Telnet

## 🔹 Telnet

- Port: 23

- Plain text

- Not secure

- Used only for testing

## 🔹 SSH (Secure Shell)

- Port: 22

- Encrypted remote login

- Used by admins, hackers, servers

SSH = Most secure remote control method.

# SMTP, POP3, IMAP

## 🔹 SMTP (Simple Mail Transfer Protocol)

- Port: 25

- Emails send করতে ব্যবহৃত হয়।

## 🔹 POP3 (Post Office Protocol v3)

- Port: 110

- Email download করে device-এ রাখে

- Server থেকে mail delete হয় (mostly)

## 🔹 IMAP (Internet Message Access Protocol)

- Port: 143

- Emails sync করে

- Server-এ কপি থাকে

- Used in modern devices

# SNMP (Simple Network Management Protocol)

Network monitoring-এর জন্য ব্যবহৃত হয়।

### SNMP কী মনিটর করে?

- Router status

- CPU usage

- Bandwidth

- Interface up/down

- Errors

- Traffic graphs

### SNMP Ports:

- 161 → Queries

- 162 → Traps (alerts)

# NTP (Network Time Protocol)

- Port: 123 (UDP)

- Time synchronization করে

- সব সার্ভার/ডিভাইসকে একই সময় দেয়

Example:
Google NTP → time.google.com

# SMB, LDAP, RDP

## 🔹 SMB (Server Message Block)

- Port: 445

- Windows File Sharing

- Used for:

  - File share

  - Printer share

  - Windows AD communication

- Attacks: EternalBlue, WannaCry

## 🔹 LDAP (Lightweight Directory Access Protocol)

- Port: 389

- Directory data store & authentication

- Used by Active Directory (AD)

## 🔹 RDP (Remote Desktop Protocol)

- Port: 3389

- Windows remote access

- Graphical remote control

- Used by admins & attackers

# NAT & PAT (Network and Port Address Translation)

## 🔹 NAT (Network Address Translation)

Private IP → Public IP
Used in routers.

উদাহরণ:

192.168.1.10 → 203.0.113.5

## 🔹 PAT (Port Address Translation)

Many private devices → One public IP
But mapped by port.

Example:

192.168.1.10:5050 → 203.0.113.5:30001

192.168.1.11:5050 → 203.0.113.5:30002

PAT = NAT Overload

Used in home routers.

# Firewalls & CDN

## 🔹 Firewall

Firewall traffic ফিল্টার করে।

### Firewall filters:

- IP

- Port

- Protocol

- Application

- URL

- Content

Types:

- Packet filtering

- Stateful firewall

- Proxy firewall

- Next-generation firewall (NGFW)

## 🔹 CDN (Content Delivery Network)

CDNs world-wide content দ্রুত পৌঁছায়।

### কাজ:

- Caching

- DDoS protection

- Load balancing

- Low latency

Examples:

- Cloudflare

- Akamai

- Fastly

# VPN (SSL, IPsec, PPTP, L2TP)

## 🔹 SSL VPN

- Works through HTTPS (443)

- User-friendly

- Used for remote workers

## 🔹 IPsec VPN

- Most secure

- Works at network layer

- Used for site-to-site VPN

## 🔹 PPTP

- Old

- Weak encryption

- Not recommended

## 🔹 L2TP/IPsec

- Combination of L2TP + IPsec

- Secure + stable

- Used widely in enterprises

# TLS (Transport Layer Security)

TLS = HTTPS নিরাপত্তার মূল প্রযুক্তি।

### TLS কী করে?

- Data encrypt করে

- Authentication দেয়

- Integrity নিশ্চিত করে

- Man-in-the-middle আক্রমণ ঠেকায়

### TLS versions:

- TLS 1.0 → Deprecated

- TLS 1.2 → Standard

- TLS 1.3 → Fastest + Most Secure

# Netstat & Traceroute

## 🔹 Netstat

Netstat = “Network Statistics”
এটি দিয়ে সিস্টেমে কি কি connections active আছে তা দেখা যায়।

### Netstat কী দেখায়?

- Active TCP/UDP connections

- Listening ports

- Established connections

- Routing table

- Network interfaces

### Common netstat outputs:

netstat -tulnp

| Flag | Meaning |
| --- | --- |
| -t | TCP |
| -u | UDP |
| -l | Listening ports |
| -n | Show numeric IP/ports |
| -p | Show process using the port |

## 🔹 Traceroute

Traceroute দেখায় একটি packet গন্তব্যে পৌঁছাতে কতগুলি রাউটার (hop) পেরোয়।

### Traceroute কীভাবে কাজ করে?

Traceroute ICMP ব্যবহার করে TTL (Time To Live) বাড়াতে বাড়াতে packet পাঠায়:

- TTL=1 → 1st router reply

- TTL=2 → 2nd router reply

- TTL=3 → 3rd router reply

এভাবে পুরো পথ দেখা যায়।

Command:

traceroute google.com

# Encryption Protocols (WEP, WPA, WPA2, WPA3)

## 🔹 WEP (Wired Equivalent Privacy)

- খুব দুর্বল

- RC4 encryption

- সহজে ভাঙা যায়

- এখন আর ব্যবহার হয় না

## 🔹 WPA (Wi-Fi Protected Access)

- WEP থেকে ভালো

- TKIP ব্যবহার করে

- দুর্বল, WPS আক্রমণে ভাঙা যায়

## 🔹 WPA2

- সবচেয়ে বেশি ব্যবহৃত

- AES-CCMP encryption

- শক্তিশালী নিরাপত্তা

## 🔹 WPA3

- নতুন স্ট্যান্ডার্ড

- SAE (Simultaneous Authentication of Equals)

- KRACK থেকে immune

- বেশি security, harder to crack

# 3. Wireless Authentication (PSK, Enterprise)

## 🔹 WPA/WPA2 PSK (Pre-Shared Key)

- Password দিয়ে WiFi connect হয়

- Home networks-এর জন্য

- Everyone uses same key

## 🔹 WPA Enterprise (802.1X)

- Username + password required

- RADIUS server authentication

- Corporate/office network-এ ব্যবহৃত

### PSK vs Enterprise Table:

| Feature | PSK | Enterprise |
| --- | --- | --- |
| Key | Shared | Per-user |
| Security | Medium | High |
| Use | Home | Company |

# Wireless Attacks (Deauth, Evil Twin, KRACK)

## 🔹 1) Deauthentication Attack

- WiFi থেকে ইউজারকে ডিসকানেক্ট করে

- Attack tool: aireplay-ng

- Purpose: Handshake capture (WPA/WPA2 cracking)

## 🔹 2) Evil Twin Attack

- Fake WiFi access point তৈরি করা

- Victim connect হলে তার ট্রাফিক attacker দেখে

- Used for: MITM, credential stealing

## 🔹 3) KRACK Attack

- WPA2 handshake vulnerability

- Replay attack

- Attacker encrypted ট্রাফিকও পড়তে পারে

# ARP Spoofing / ARP Poisoning

ARP Spoofing = attacker নিজের MAC address-কে gateway বা victim-এর IP-এর সাথে bind করে।

### ফলাফল:

- MITM attack

- Data sniffing

- Session hijacking

- DNS spoofing possible

### Concept:

Attacker:  "I am the gateway"

Victim → sends packets to attacker

Tools: ettercap, bettercap, arpspoof

# DNS Spoofing

DNS spoofing = attacker fake DNS response পাঠায় যাতে victim ভুল IP-তে যায়।

### Example:

facebook.com → 192.168.1.50 (attacker server)

Used for:

- Phishing

- Redirection

- MITM attacks

# IP Spoofing

Attacker fake source IP ব্যবহার করে packet পাঠায়।

### Uses:

- Hiding identity

- SYN flood attack

- Bypassing IP filtering

- Smurf attack

Since IP spoofing changes source IP, victim reply wrong host.

# MAC Flooding

Switch-এর MAC table flood করা হয় random MAC দিয়ে।

### Effect:

- Switch becomes full

- Switch turns to hub mode (broadcast everything)

- Attacker can sniff traffic

Tool: macof (from Dsniff)

# DHCP Starvation

Attacker অনেক fake DHCP requests পাঠায়, DHCP server-এর IP pool শেষ হয়ে যায়।

### Result:

- Legit users get no IP

- Attacker starts Rogue DHCP server

Used for:

- MITM

- Redirection attack

Tools: Yersinia

# SYN Flood / DoS / DDoS

## 🔹 SYN Flood

Attacker প্রচুর SYN পাঠায় server-এ
Server SYN/ACK পাঠায়
Client ACK কখনো পাঠায় না

### Result:

- Server half-open connection table পূর্ণ

- Server freeze / crash

## 🔹 DoS

Single attacker → target down করে।

## 🔹 DDoS

Many attackers → huge traffic → server down
Uses botnets (Mirai, Zeus)

# MITM (Man-in-the-Middle)

MITM = attacker দুই পক্ষের মাঝখানে থেকে ডাটা change বা capture করে।

### MITM techniques:

- ARP Spoofing

- DNS Spoofing

- Evil Twin

- HTTPS downgrade attack

- SSL stripping

### What attacker can do?

- Read traffic

- Modify traffic

- Inject malware

- Hijack sessions

# Sniffing & Eavesdropping

## 🔹 Sniffing

Sniffing মানে নেটওয়ার্কের মধ্যে চলাচল করা ডাটা capture করা।

### Passive Sniffing

- Network listen করে

- Victim বুঝে না

- Switch networks-এ কঠিন

### Active Sniffing

- Attacker actively packet manipulate করে

- ARP Spoofিং

- MAC Flooding

- DHCP Spoofing

### Tools:

- Wireshark

- tcpdump

- tshark

- Ettercap / Bettercap

## 🔹 Eavesdropping

Eavesdropping = গুপ্তভাবে কথা শোনা।

নেটওয়ার্কে:

- HTTPS না থাকলে attacker ডাটা পড়তে পারে

- যেমন: username/password

এটা sniffing-এরই কোভার্ট ফর্ম।

# Replay Attacks

Attacker কোনো বৈধ প্যাকেট capture করে আবার resend করে।

### উদাহরণ:

- Login request capture → পরে পাঠানো

- Payment request replay

- Authentication token reuse

### প্রতিরোধ:

- Time-stamps

- Nonce

- TLS/HTTPS

# Session Hijacking

Attacker একটি active session দখল করে নেয়।

### ধাপ:

- Attacker session cookie sniff করে

- Cookie ব্যবহার করে victim-এর account takeover

### Types:

- Active hijacking: attacker packet modify করে

- Passive hijacking: শুধু session পড়ে

- Cookie hijacking: session id চুরি

### Tools:

- Burp Suite

- Bettercap

- Wireshark

# ICMP Flood / Ping of Death / Smurf Attack

## 🔹 ICMP Flood

Attacker সার্ভারকে অনেক ICMP Echo (ping) পাঠায়।

Result:

- Server CPU overload

- ICMP stack ব্যস্ত হয়ে যায়

- DoS ঘটে

## 🔹 Ping of Death

- Huge ICMP packet পাঠিয়ে system crash করা

- Old OS vulnerable ছিল

- Modern OS patched

## 🔹 Smurf Attack

- Attacker → broadcast address-এ ICMP পাঠায়

- Source IP spoof করে victim-এর

- Network → সব reply victim-এ পাঠায়

- Victim overloaded হয়ে যায়

# VLAN Hopping

Attacker এক VLAN থেকে অন্য VLAN-এ access পায়।

### Techniques:

- Switch Spoofing

  - Attacker নিজের ডিভাইসকে Trunk port-এর মতো behave করায়

- Double Tagging

  - Double 802.1Q tag ব্যবহার করে VLAN bypass

### Prevent:

- Disable DTP

- Set all ports to access

- Use VLAN pruning

# Rogue DHCP & Evil Twin

## 🔹 Rogue DHCP

Attacker fake DHCP server বানায়।

### Result:

- Victim wrong gateway পায়

- Traffic attacker-এর কাছে যায়

- MITM possible

- DNS spoofing possible

## 🔹 Evil Twin

Fake WiFi Access Point বানানো।

### Works:

- Victim thinks it's real

- Connects

- Attacker sniffs + controls traffic

# Routing Attacks

Routing protocol manipulate করার আক্রমণ।

### Types:

### 1) Route Injection

- Fake route advertise করা

- Traffic attacker-এর দিকে reroute

### 2) Blackhole Attack

- Traffic drop করে

### 3) Routing Table Poisoning

- Wrong entries দিয়ে routing নষ্ট করা

### Protocols targeted:

- OSPF

- BGP

- RIP

# SSL Stripping

HTTPS → HTTP করে ফেলা (downgrade attack)।

### কাজ:

- Victim → HTTPS request

- Attacker intercept করে

- Server-এর সাথে HTTPS

- Victim-এর সাথে HTTP

- Attacker সব data পড়ে + পরিবর্তন করতে পারে

Used in WiFi MITM attacks.

Tools:

- sslstrip

- Bettercap

- Burp Suite

# Port Scanning Detection & Prevention

## 🔹 Detection:

- IDS/IPS (Snort, Suricata)

- Firewall logs

- Unusual SYN packets

- nmap fingerprinting detection

## 🔹 Prevention:

- Firewall rules tighten

- SYN cookies enable

- Port knocking

- Disable unused ports

- Rate limiting

# Pivoting Through Networks

Pivoting = এক ডিভাইস compromise করার পর সেখান থেকে অন্য নেটওয়ার্ক/ডিভাইসে প্রবেশ।

### Techniques:

- SSH Tunneling

- Meterpreter pivot route

- SOCKS proxy

- Proxychains

- VPN pivot

Example:

Attacker → Compromised PC → Internal Servers

Used in post-exploitation.

# Traffic Redirection & Tunneling

## 🔹 Traffic Redirection

Attacker network traffic অন্য জায়গায় পাঠায়:

- DNS Spoofing

- ARP Poisoning

- Route Injection

## 🔹 Tunneling

এক প্রোটোকলের ভিতরে আরেক প্রোটোকল লুকানো।

Examples:

- SSH Tunnel

- HTTP Tunnel

- ICMP Tunnel

- DNS Tunnel

Used to bypass firewalls.

# Reverse Shells & Bind Shells

## 🔹 Reverse Shell

Victim attacker-কে connect করে।
(Easiest to bypass firewall)

Victim → Attacker listener

Used in:

- Metasploit

- Netcat

- Python payloads

## 🔹 Bind Shell

Victim machine একটি port খুলে, attacker সেখানে connect করে।

Attacker → Victim open port

Harder to bypass firewall (needs open port).